Rolls-Royce 40/50 Silver Ghost Chassis Series Information

Number produced: 6173

Original Specification:

6 cylinders (2 blocks of 3), side valves, 4.5” x 4.5” (7036 c.c.), cone clutch, 4 speeds, dual ignition with magneto and trembler coil, footbrake external contacting on propeller shaft, hand-brake internal expanding on rear drums, suspension semi-elliptic front and platform rear, wheelbase 135 ½" or 143 ½", chassis weight 2050 lbs. or 2200 lbs., tyres 875 x 105 (f) and 880 x 120 (f) and 895 x 135 (r).

Chassis Series	Year		Changes Introduced

60539-60592	1907		60551 Silver Ghost (AX201)

60581 Semi-elliptic rear suspension

60700-60799	1907/8

919-1015		1908/9	Friction shock absorbers

1100-119		1909/10	3 speed gearbox introduced, mechanical air

pump, 4.5” x 4.75” (7428 c.c.)

1200-1299		1910

1300-1399		1910

1400-1499		1910/11

1500-1699		1911		Vibration damper

1700-1799		1911		Torque tube axle

1800-1999		1912

2000-2699		1912/13	Cantilever springs from 2100 on, 4 speed

Gearbox introduced at 2400, Double rear wheel

brakes, chassis tie rods.

CA 1-20		1913

NA 1-58		1913/14

MA 1-56		1914

AB 1-67		1914

EB 1-60		1914		3 speeds discontinued, 895 x 135 tyres

RB 1-68		1914

PB 1-65		1914		Large carburettor standardised, threaded union

YB 1-66		1914

LB 1-68		1914

GB 1-49		1914

TB 1-37 & 55	1914/15

BD 1-32		1915

AD 1-32		1915

ED 1-34		1915

RD 1-35		1915/16

CB 1-37		1915

PD 19-33		1915

AC 1-28		1915		Modified chassis

X 1-16			1919/20/21	Show models only

PP 1-36		1919		Air pump on governor casing, 5 ½" radiator,

electric starter and lighting, non-trembler coil,

priming device

LW 1-48		1919

TW 1-81		1920

CW 1-102		1920

FW 1-121		1920

BW 1-165		1920

AE 1-141		1920

EE 1-141		1920

RE 1-81		1920

PE 1-81		1920

YE 1-81		1921

UE 1-81		1921

LE 1-81		1921

WO 1-279		1920/27	Armoured car chassis

GE 1-81		1920/21

TE 1-81		1921

CE 1-107		1921		Some numbers used by Springfield

NE 1-123		1921		" "

AG 1-182		1921/22	" "

LG 1-198		1921		" "

UG 1-97		1921/22	" "

SG 1-91		1921		" "

TG 1-94		1922		" "

KG 1-43		1922		Springfield used 276KG to 400KG

PG 1-44		1922

RG 1-43		1922

YG 1-81		1922

ZG 1-81		1922/23

HG 1-102		1922/23

LK 1-100		1923		51-100 duplicated by Springfield with "S" prefix

NK 1-100		1923

PK 1-63		1923		Springfield used 201PK to 300PK

EM 1-135		1924/24	Front-wheel brakes

LM 1-71		1924

RM 1-103		1924		Late cars were of 144" or 150 ½" wheelbase,

had 33 x 5 tyres and Autovac

TM 1-103		1924

AU 1-141		1924/25

EU 1-129		1925

END